Lab #3 - Questions 2 and 3

Question 2: Multiple-of-3-or-5 Detector

Let A, B, C, and D be a 4-bit signed 2's complement input, where A is the sign bit. The possible values are -8 through +7. The output F is 1 when the input is a multiple of 3 or 5.

The values that make F = 1 are: -6, -5, -3, 0, 3, 5, and 6.

Therefore: F(A,B,C,D) = Σm(0, 3, 5, 6, 10, 11, 13).

Truth Table

| A | B | C | D | Decimal | F |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 0 | 1 | 1 | 0 |
| 0 | 0 | 1 | 0 | 2 | 0 |
| 0 | 0 | 1 | 1 | 3 | 1 |
| 0 | 1 | 0 | 0 | 4 | 0 |
| 0 | 1 | 0 | 1 | 5 | 1 |
| 0 | 1 | 1 | 0 | 6 | 1 |
| 0 | 1 | 1 | 1 | 7 | 0 |
| 1 | 0 | 0 | 0 | -8 | 0 |
| 1 | 0 | 0 | 1 | -7 | 0 |
| 1 | 0 | 1 | 0 | -6 | 1 |
| 1 | 0 | 1 | 1 | -5 | 1 |
| 1 | 1 | 0 | 0 | -4 | 0 |
| 1 | 1 | 0 | 1 | -3 | 1 |
| 1 | 1 | 1 | 0 | -2 | 0 |
| 1 | 1 | 1 | 1 | -1 | 0 |

K-Map

| AB \ CD | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | 1 | 0 | 1 | 0 |
| 01 | 0 | 1 | 0 | 1 |
| 11 | 0 | 1 | 0 | 0 |
| 10 | 0 | 0 | 1 | 1 |

Using the K-map groups (m10,m11), (m3,m11), and (m5,m13), with m6 and m0 remaining as single cells, the minimized expression is:

F = AB'C + B'CD + BC'D + A'BCD' + A'B'C'D'

Circuit Diagram

AB'C -----------\

B'CD ------------\

BC'D -------------[ OR ] ---- F

A'BCD' ----------/

A'B'C'D' -------/

The required complemented inputs A', B', C', and D' are produced with NOT gates. Each product term is implemented with an AND gate, and all five AND outputs are connected to the OR gate.

Question 3: 4-Bit Signed Multiplier Using Booth's Algorithm

The multiplier accepts two 4-bit signed 2's complement numbers from -8 to +7. Booth's algorithm can be used to perform the signed multiplication. The design can be divided into three major functional components.

1. Registers

Use a 4-bit M register for the multiplicand, a 4-bit Q register for the multiplier, a 4-bit A register for the partial product, and a 1-bit Q-1 register. Initially, A = 0000 and Q-1 = 0. After four Booth cycles, A and Q are concatenated to form the product.

2. 4-Bit Adder/Subtractor

The adder/subtractor updates A by performing A + M or A - M. Subtraction can be implemented with 2's complement, so the same adder hardware can perform both operations.

3. Booth Control and Arithmetic Shift Logic

The control logic examines Q0 and Q-1 to select the required operation. After the operation, [A,Q,Q-1] is shifted one bit to the right using an arithmetic right shift. The process repeats four times.

Booth Control Table

| Q0 | Q-1 | Operation |
| --- | --- | --- |
| 0 | 0 | No operation |
| 0 | 1 | A = A + M |
| 1 | 0 | A = A - M |
| 1 | 1 | No operation |

Block Diagram

Multiplicand ---> [ M Register ] ---> [ Adder/Subtractor ] ---> [ A Register ]

^                       |

|                       v

[ Booth Control ] <--- [ Q Register ] <--- Multiplier

^                       |

|                       v

+------------------- [ Q-1 ]

After each cycle: Arithmetic right shift [A,Q,Q-1]

Final output after four cycles: [ A (4 bits) | Q (4 bits) ]

Output Size

The possible product range is -56 to +64. A 7-bit signed number represents only -64 to +63, so it cannot represent +64. Therefore, an 8-bit signed output is required.

Required output size: 8 bits.